PCV Specification

| Item | Materials |
| --- | --- |
| Footing | Precast Rcc footing, Tie beam with 2200 psi grade concrete. |
| Light Steel Materials | Column, Rafter, Purlin and others member made of MS steel Grade SS400, strength 250 Mpa |
| Ground floor | 24’’x24’’x2.5’’precast cement concrete block is used |
| Outer wall | 2.5’’ thickness RC precast cocreate panel |
| Inside wall | 2’’ thickness RC precast concrete panel |
| Roof | 0.4mm Color steel sheet has to be used |
| Door | 7’x3’ Steel frame with single panel shutter. |
| Toilet & Kitchen door | 7’x2.5’ Steel frame with single panel shutter. |
| Window | 4’x3’ Steel frame and steel shutter with Grill |
| Floor finishing | Net Cement finishing |
| Celling finishing | PVC board |
| Bath room fitting | PVC fitting |
| Electric wiring | 2 bulb, 1 fan point, 2 power sockets in each room. BRB cable is to be used. |

PDV Specification

| Item | Materials |
| --- | --- |
| FOOTING | Precast RC footing &Tie beam with 2200 psi concrete is to be used. |
| LIGHT STEEL STRUCTURE | Column, Rafter, Purlin and others member made of MS steel Grade SS400, strength 250 Mpa |
| FLOOR | 24’’x24’’x2.5’’precast cement concrete block is used |
| OUTER WALL | 2.5’’ thickness RC precast cocreate panel |
| INNER WALL | 2’’ thickness RC precast concrete panel |
| ROOF | 0.4mm color steel sheet with 8mm heat insulation. |
| DOOR | 7’x3’ Steel frame with wooden shutter |
| TOILET & KITCHEN DOOR | 7’x2.5’ PVC frame with PVC shutter |
| WINDOW | 4’x4.5’ Thai frame, Glass shutter with Grill |
| FLOOR FINISHING | 16’’x16’’ Tiles finishing |
| PAINT | Weather coat and plastic paint |
| CEILING | PVC board with Thai frame |
| BATHROOM FITTING | Metal fitting |
| ELECTRIC WIRING | 3 bulb, 1 fan point, 3 power sockets in each room. BRB cable is to be used. |

Prefab Building Specification

| Item | Materials |
| --- | --- |
| FOOTING | Precast RC footing &Tie beam with 2200 psi concrete. |
| LIGHT STEEL STRUCTURE | Column, rafter, purlin and others member made of MS steel grade SS400, strength 250 Mpa |
| GROUND FLOOR | 3’’ thickness cement concrete. |
| OUTER WALL | 2.5’’ thickness RC precast cocreate panel |
| INNER WALL | 2’’ thickness RC precast concrete panel |
| MIDLE & ROOF FLOOR | 3’’ thickness RC precast concrete panel |
| DOOR | 7’x3.5’ steel frame with wooden shutter |
| TOILET & KITCHEN DOOR | 7’x2.5’ PVC frame with PVC shutter |
| WINDOW | 5’x4.5’ Thai frame, Glass shutter with Grill |
| FLOOR FINISHING | 16''x16'' Tiles |
| PAINT | Weather coat and plastic paint |
| ROOF TOP | 1’’ cement mortar finishing |
| BATHROOM FITTING | Metal fittings |
| ELECTRIC WIRING | 3 bulb,1 fan point, 3 power sockets in each room. BRB cable is to be used. |

| Dimension | Quantity |
| --- | --- |
| Floor area | 5000 sft |
| Length | 100 ft |
| Width | 50 ft |
| Eave height | 15 ft |
| Mid height | 20 ft |
| Side wall | 8 ft |
| Drop wall | 7 ft |